{if $DocumentType eq ‘Report’}

{$ProjectTitle}

{if $DocumentTitle eq ‘Custom Title of Document’}{$CustomTitle}{else}{$DocumentTitle}{/if}

{$ProjectNumber}RP{$DocumentNumber} Revision {$RevisionNumber}

Tuesday, 2 December 2025

# Document Information

| Project | {$ProjectTitle} |
| --- | --- |
| Client | {$Client} |
| Report title | {if $DocumentTitle eq ‘Custom Title of Document’}{$CustomTitle}{else}{$DocumentTitle}{/if} |
| Project Number | {$ProjectNumber} |

Revision Table

| Report revision | Date | Description | Author | Reviewer |
| --- | --- | --- | --- | --- |
| {$RevisionNumber} |  | {$RevisionText} | {$Author} | {$Reviewer} |

Disclaimer

This report has been prepared by Resonate Consultants Pty Ltd (Resonate) for the exclusive use of our Client. Our advice is not intended for use by any third parties, and any reliance on our advice by third parties shall be entirely at their own risk. Resonate accepts no responsibility or liability for any consequences arising from the use of our advice by persons other than our Client. Our advice has been prepared for the specific purpose and scope agreed with our Client. It is not intended to be a substitute for professional advice in other contexts or to address other issues outside the scope of work for this project.

The information, findings, and recommendations are based on the conditions and data available at the time of preparation. Any opinions or recommendations expressed are subject to the assumptions, limitations, and conditions as stated. Any reliance on external information has been accepted in good faith as being accurate and valid.

Glossary

| A-weighting | A spectrum adaption that is applied to measured noise levels to represent human hearing. A-weighted levels are used as human hearing does not respond equally at all frequencies. |
| --- | --- |
| Characteristic | Associated with a noise source, means a tonal, impulsive, low frequency, intermittent, or modulating characteristic of the noise that is determined in accordance with the Guidelines for the use of the Environment Protection (Commercial and Industrial Noise) Policy 2023 to be fundamental to the nature and impact of the noise. |
| Continuous noise level | A-weighted noise level of a continuous steady sound that, for the period over which the measurement is taken using fast time weighting, has the same mean square sound pressure as the noise level which varies over time when measured in relation to a noise source and noise-affected premises in accordance with the Noise Policy |
| Day | Between 7 am and 10 pm as defined in the Noise Policy |
| dB | Decibel—a unit of measurement used to express sound level. It is based on a logarithmic scale which means a sound that is 3 dB higher has twice as much energy. We typically perceive a 10 dB increase in sound as a doubling of loudness. |
| dB(A) | Units of the A-weighted sound level. |
| Frequency (Hz) | The number of times a vibrating object oscillates (moves back and forth) in one second. Fast movements produce high frequency sound (high pitch/tone), but slow movements mean the frequency (pitch/tone) is low. 1 Hz is equal to 1 cycle per second. |
| Indicative noise level | Indicative noise level determined under clause 5 of the Noise Policy. |
| L90 | Noise level exceeded for 90 % of the measurement time. The L90 level is commonly referred to as the background noise level. |
| Leq | Equivalent Noise Level—Energy averaged noise level over the measurement time. |
| Lmax | The maximum instantaneous noise level. |
| Night | Between 10 pm on one day and 7 am on the following day as defined in the Noise Policy |
| Noise source | Means a commercial or industrial premises at which an activity is undertaken, or a machine or device is operated, resulting in the emission of noise |
| Quiet noise designated area | A noise designated area is a quiet noise designated area if the land uses under the Planning and Design Code provisions that make land use rules for the noise designated area are principal land uses that all fall within either or both of the following land use categories (a) Residential; (b) Rural Living. |

# Table of Contents

Word did not find any entries for your table of contents.
In your document, select the words to include in the table of contents, and then on the Home tab, under Styles, click a heading style. Repeat for each heading that you want to include, and then insert the table of contents in your document. To manually create a table of contents, on the Document Elements tab, under Table of Contents, point to a style and then click the down arrow button. Click one of the styles under Manual Table of Contents, and then type the entries manually.

{else}Tuesday, 2 December 2025

Project number: {$ProjectNumber}

Reference: {$ProjectNumber}LT{$DocumentNumber}

{$ClientName}

{$CompanyName}

{$CompanyAddress}

Dear {$ClientFirstName},

{$ProjectName}

{if $DocumentTitle eq ‘Custom Title of Document’}{$CustomTitle}{else}{$DocumentTitle}{/if}{/if}

#### Executive summary

This report outlines the environmental noise assessment for the proposed xxx Development, located at yyy.

The potential noise emissions from the development have been assessed to adjacent noise-affected premises and potential noise sensitive receptors on the development site itself.

The potential environmental noise impact has been assessed against the:

Planning & Design Code

Environment Protection (Commercial & Industrial Noise) Policy 2023 (Noise Policy)

Local Nuisance and Litter Control Act 2016

World Health Organization (WHO) Guidelines for Community Noise

Environment Protection Authority (EPA) Guideline Assessing music noise from indoor venues (2021)

Measured existing noise levels on site/at a location representative of the most noise-affected premises.

A summary of the noise assessment and outcomes are outlined in Table 1.

Table 1 Assessment summary and outcomes

| Noise source | Outcome |
| --- | --- |
| Childcare centre | No specific acoustic treatments are required |
| Car parking activity | No specific acoustic treatments are required. |
| Loading and deliveries | - Roller doors to Loading & Deliveries / Waste Collection area to be closed during use of the loading area. - No refrigerated truck deliveries. - Frequency of deliveries limited to 1 delivery in a 15-mintue period. - Electric forklift or hand pallet jack to be used for unloading |
| Building services plant | At this stage of the development process, detailed information on the Building Services equipment is not available. Potential noise mitigation treatments that will be considered during detailed design are provided in the report. During the detailed design phase, noise emissions from the external plant will be assessed and mitigation treatments adopted to ensure that cumulative noise emissions are limited to 52 dB(A) during the day (7 am to 10 pm) and 45 dB(A) at night (10 pm to 7 am) at the noise-affected premises, when assessed in accordance with the South Australian Environment Protection (Commercial & Industrial Noise) Policy 2023. |
| Waste collection | Waste collection is to be restricted to 9 am and 7 pm on a Sunday or public holiday and 7 am and 7 pm on any other day, |

This assessment has demonstrated that the potential noise emissions from the operation of the proposed development will be able to operate within the relevant noise provisions in the Planning & Design Code and relevant noise criteria requirements.

# Introduction

This report outlines the environmental noise assessment for the proposed [development name] at [location]. {$ProjectDescription} The closest noise-affected premises are located [description of locality in relation to development].

The noise impact assessment has been undertaken in consideration of the following guidelines:

Planning & Design Code

Environment Protection (Commercial & Industrial Noise) Policy 2023 (Noise Policy)

Local Nuisance and Litter Control Act 2016

World Health Organization (WHO) Guidelines for Community Noise

Environment Protection Authority (EPA) Guideline Assessing music noise from indoor venues (2021).

# Proposed development

## Location

The subject site is located at [location] and is bounded by industrial development to the north and east, Torrens Road to the south west, and a substation to the north west. The closest noise-affected premises are located approximately 50 m across Torrens Road along Pyres Street and O'Neil Lane.

Figure 1 shows an aerial image of the site in relation to adjacent land and zoning.

Figure 9 Aerial image of site, adjacent land, and zoning

## Operation

It is proposed to have the following hours of operation…, with the activity as outlined in Table 2.

Table 2 Proposed activity

| Noise sources | Location | Period |  |
| --- | --- | --- | --- |
|  |  | Day—7 am to 10 pm | Night—10 pm to 7 am |
| General operations |  |  |  |
| Warehouse operations | Main warehouse | - Dry goods storage only - Electric forklift movements - Assumed internal noise level of LP 75 dB(A). No hearing protection required for staff | No nighttime operations |
| Building services plant |  |  |  |
| - Condensers - Air handling units - Cooling towers - Chillers - Heat exchangers - Pumps - Relief / extraction / outside air fans | Roof top enclosed plant area | All plant operating at 100% capacity | No nighttime operation |
| Refrigeration plant | Plant deck on roof | Refrigeration plant operating at 100% capacity | Refrigeration plant operating at 100% capacity |
| Emergency equipment: - Standby generator - Fire pump - Smoke Spill Fans - Stair Pressurisation Fans (buildings ≥ 25m) | Plant deck on roof | - Tested monthly for up to 15 minutes each - Full operation as required during an emergency | Full operation as required in an emergency |
| Vehicle movements |  |  |  |
| Car park activity | Basement to L2 with entry from Leader Street | 20 car movements per 15-minute period | 5 car movements per 15-minute period |
| Loading dock activity | On grade to south of development with access from Leader Street | - 1 delivery with refrigerated HRV per 15-minute period. - No idling while unloading - Electric forklift unloading | No nighttime deliveries |
| Music noise |  |  |  |
| Music noise | Main function room | Background music only | Amplified music—Typical function room levels at 93 dB(A) L10 until 2 am the following evening for all nights. |
| Music noise | Outdoor beer garden | Background music only | Background music only |
| People noise |  |  |  |
| Childcare centre: - 10 children 0 – 2 years - 20 children 2 – 3 years - 30 children 3 – 5 years | Outdoor play area | All using outdoor areas (split into areas?) | No nighttime operations |

## Proposed construction

What are the proposed facade or barrier constructions if relevant.

# Commercial & Industrial—Planning & Design Code

## Zoning

The zoning of the subject site (noise source) and noise-affected premises are as outlined in Table 17.

Table 3 Summary of zones/subzones

| Location | Zone / Subzone |
| --- | --- |
| Subject site | Urban Corridor Living Retail |
| Premises to the north along Maple Avenue | Urban Corridor Living Retail |
| Premises to the east | Urban Corridor (Business) |
| Premises to the south along Leader Street | Established Neighbourhood |

The Desired Outcomes, Performance Outcomes, and Designated Performance Features of the zones/subzones have been used by the EPA in their Indicative Noise Factor Guidelines to determine appropriate land use categories and indicative noise factors used to derive environmental noise criteria (refer to Section 0 for further details).

## Interface between land uses

Interface between Land Uses is a General Development Policy that is relevant to the subject site. The relevant Assessment Provisions relating to noise are outlined in Table 4.

Table 4 Relevant Assessment Provisions—Activities generating noise or vibration

| Relevant Assessment Provisions |  |
| --- | --- |
| Desired Outcome |  |
| DO1 | Development is located and designed to mitigate adverse effects on or from neighbouring and proximate land uses. |
| Performance Outcome | Deemed-to-Satisfy Criteria / Designated Performance Feature |
| PO 4.1 Development that emits noise (other than music) does not unreasonably impact the amenity of sensitive receivers (or lawfully approved sensitive receivers). | DTS/DPF 4.1 Noise that affects sensitive receivers achieves the relevant Environment Protection (Commercial and Industrial Noise) Policy criteria. |
| PO 4.2 Areas for the on-site manoeuvring of service and delivery vehicles, plant and equipment, outdoor work spaces (and the like) are designed and sited to not unreasonably impact the amenity of adjacent sensitive receivers (or lawfully approved sensitive receivers) and zones primarily intended to accommodate sensitive receivers due to noise and vibration by adopting techniques including: - locating openings of buildings and associated services away from the interface with the adjacent sensitive receivers and zones primarily intended to accommodate sensitive receivers - when sited outdoors, locating such areas as far as practicable from adjacent sensitive receivers and zones primarily intended to accommodate sensitive receivers - housing plant and equipment within an enclosed structure or acoustic enclosure - providing a suitable acoustic barrier between the plant and / or equipment and the adjacent sensitive receiver boundary or zone. | DTS/DPF 4.2 None are applicable. |
| PO 4.3 Fixed plant and equipment in the form of pumps and/or filtration systems for a swimming pool or spa are positioned and/or housed to not cause unreasonable noise nuisance to adjacent sensitive receivers (or lawfully approved sensitive receivers). | DTS/DPF 4.3 The pump and/or filtration system ancillary to dwelling erected on the same site is: - enclosed in a solid acoustic structure located at least 5m from the nearest habitable room located on an adjoining allotment or - located at least 12m from the nearest habitable room located on an adjoining allotment. |
| PO 4.4 External noise into bedrooms is minimised by separating or shielding these rooms from service equipment areas and fixed noise sources located on the same or an adjoining allotment. | DTS/DPF 4.4 Adjacent land is used for residential purposes. |
| PO 4.5 Outdoor areas associated with licensed premises (such as beer gardens or dining areas) are designed and/or sited to not cause unreasonable noise impact on existing adjacent sensitive receivers (or lawfully approved sensitive receivers). | DTS/DPF 4.5 None are applicable. |
| PO 4.6 Development incorporating music achieves suitable acoustic amenity when measured at the boundary of an adjacent sensitive receiver (or lawfully approved sensitive receiver) or zone primarily intended to accommodate sensitive receivers | DTS/DPF 4.6 Development incorporating music includes noise attenuation measures that will achieve the following noise levels: Assessment location; Music noise level. Externally at the nearest existing or envisaged noise sensitive location; Less than 8dB above the level of background noise (L90,15min) in any octave band of the sound spectrum (LOCT10,15 < LOCT90,15 + 8dB). |

# Commercial & Industrial—Noise criteria

## Environmental noise policy

As noted in DTS/DPF 4.1, environmental noise emissions from the subject site should comply with the Environment Protection (Commercial & Industrial Noise) Policy 2023 (Noise Policy).

As noted in Section 3.1, the environmental noise criteria are based on the land use categories determined based on the principal land uses outlined in the Planning & Design Code for the noise source and noise-affected premises. The land uses and indicative noise factors are as shown in Table 5 and Table 6. Note that the indicative noise factors in Table 5 are used where the noise source and noise affected premises falls within the same land use category (being only General Industry and Special Industry). In all other cases the indicative noise factors in Table 6 are to be used.

Table 5 Excerpt from Noise Policy—Table 1(subclause(1)(a))

| Land use category | Indicative noise factor dB(A) |  |
| --- | --- | --- |
|  | Day (7 am to 10 pm) | Night (10 pm to 7 am) |
| General industry | 65 | 65 |
| Special industry | 70 | 70 |

Table 6 Excerpt from Noise Policy—Table 2(subclause(1)(b))

| Land use category | Indicative noise factor dB(A) |  |
| --- | --- | --- |
|  | Day (7 am to 10 pm) | Night (10 pm to 7 am) |
| Rural living | 47 | 40 |
| Residential | 52 | 45 |
| Rural industry | 57 | 50 |
| Light industry | 57 | 50 |
| Commercial | 62 | 55 |
| General industry | 65 | 55 |
| Special industry | 70 | 60 |

Based on the land use categories as outlined in the EPA’s Indicative Noise Factor Guidelines, the relevant criteria for the noise-affected premises in each zone are as outlined in Table 7. In accordance with Part 5 of the Noise Policy, the relevant criteria are the average of the relevant indicative noise factors less 5 dB(A).

Table 7 Summary of zones, land uses, and Noise Policy criteria

| Site | Zone/subzone | Land use categories | Criteria, LAeq |  |
| --- | --- | --- | --- | --- |
|  |  |  | Day (7 am to 10 pm) | Night (10 pm to 7 am) |
| Subject site | Employment | Light Industry & Commercial | N/A | N/A |
| Premises to the south along Mascotte Street | Employment | Light Industry & Commercial | 55 | 48 |
| Premises to the west along Victoria Road | Neighbourhood zone | Residential & Commercial | 53 | 46 |

Additionally, as the noise-affected premises is situated in a residential and/or rural living land use category it is in a  'quiet noise designated area’, a maximum noise criterion of Lmax 60 dB(A) at night, 10 pm to 7 am, is applicable.

For information, the land use categories as nominated in Table 7 are presented in Figure 2.

Figure 10 Land use categories

## Schools, kindergartens, child-care centre of place of worship

It should be noted that the Noise Policy specifically excludes ‘noise from activities carried on in the ordinary course of the operation of a school, kindergarten, child-care centre of place of worship’. The Guidelines for the use of the Environment Protection (Commercial and Industrial Noise) Policy 2023 (the Guidelines) further notes that:

Child-care centres, schools, kindergartens, places of worships and playgrounds are often located immediately adjacent to sensitive land uses. Activities carried on in the ‘ordinary course’ of the operation normally include people noise (such as noise from school activities) and worship activities (such as music and church bells). However, other noise sources that can be assessed under the noise policy from these areas are mechanical noise sources that may be utilised on site (such as noise from an air conditioner plants, or pumps).

As such, we recommend that noise from mechanical plant be assessed under the Noise Policy, while noise from students is assessed separately in accordance with the World Health Organization (WHO) Guidelines for Community Noise. The Guidelines note that the Noise Policy indicative noise levels are ‘underpinned by the recommendations in the World Health Organization (WHO) guidelines’, and so adoption of the WHO criteria is appropriate and aligns with the intent of the Noise Policy requirements.

## WHO Guidelines for Community Noise

As discussed above, the WHO Guidelines can be used to consider the noise generated by students. It states the following with respect to noise annoyance during the daytime for outdoor areas of residences:

To protect the majority of people from being seriously annoyed during the daytime, the sound pressure level on balconies, terraces and outdoor living areas should not exceed 55 dB LAeq for a steady continuous noise. To protect the majority of people from being moderately annoyed during the daytime, the outdoor sound pressure level should not exceed 50 dB LAeq.

Based on the above, we recommend that the average noise level in the outdoor areas at the closest noise sensitive premises be no greater than 50 dB(A) LAeq,16h. Achievement of this external noise criterion will also result in the relevant WHO indoor noise criteria (35 dB(A) LAeq,16h) being achieved in residences with open windows.

## Internal noise criteria

There are certain situations in which it is more appropriate to consider the internal noise levels rather than external noise levels. Under Part 3 Clause 11 of the Noise Policy, the hierarchy for a relevant measurement location as follows:

Inside, if acoustic treatments have been applied to the building or habitable room

Outside, away from other vertical reflecting surfaces

Outside, close to reflecting surfaces, for passive recreation areas only

Inside, at the plane of an opening

Inside a room with the openings into that room closed.

Specifically, Part 3, Clause 11(5) of the Noise Policy states:

If the noise affected premises has existing noise attenuation installed, or if the measurement place cannot be located at an open window of a habitable room, the measurement place must be within such a room.

The receiver at xx has been designed to comply with the requirements of Ministerial Building Standard MBS 010 Construction requirements for the control of external sound. As such, it is considered that the building has incorporated acoustic treatments to the facade (incorporated during design as required by Development Authorisation; refer to planning application number 020/A016/16).

Part 1, Clause 5(8) provides guidance on determining appropriate internal indicative noise levels:

(a)	the lowest end of the design sound level range set out in Australian and New Zealand Standard AS/NZS 2107:2016: Acoustics — Recommended design sound levels and reverberation times for building interiors, determined by the Authority to be the relevant level; or

(b)	20 dB(A) less than the indicative noise level that would, but for this subclause, apply,

whichever is the greater.

The lowest end of the design sound level range recommended in AS/NZS 2107 for sleeping areas and living areas (for apartments in inner city areas or entertainment districts or near major roads) is Leq 35 dB(A). On this basis, and inconsideration of Part 5 of the Noise Policy, the internal noise criteria for sleeping areas and living areas for the receiver at xx is Leq 30 dB(A).

## Noise characteristics

Under the Noise Policy, noise that exhibits annoying characteristics attracts a penalty to account for such noise being more noticeable and annoying when compared to constant, broadband or steady noise. For a penalty to be applied, the characteristic must dominate the overall noise impact at the receiver, rather than being part of it.

Table 2 outlines the noise characteristics and their subjective test as outlined under the Guidelines for the use of the Environment Protection (Commercial and Industrial Noise) Policy 2023 (the Guidelines).

Table 2 Subjective tests for noise characteristics

| Noise characteristic | Subjective test under the Guidelines(1) |
| --- | --- |
| Low frequency | A noise source that attracts a low frequency characteristic will often be described as something with a rumbling, rolling, roaring or booming noise that is clearly audible above everything else. Some examples include the noise from a freight train locomotive as it moves away from a resting position, a jet aircraft taking off or the rumbling sound of thunder in nature. |
| Tonal | A noise source that attracts a tonal characteristic will often be described as something with a pitch or sharply defined note that is clearly audible above everything else. Some examples include the noise from tonal reversing signal on a truck, a whistle, the squealing generated by train wheels going around curves, or the distinctive note that can be heard from an unmuffled high performance motor held at constant speed. Note that the reversing signal on a truck may attract a penalty for a modulating characteristic due to the rise and fall of sound and a penalty for a tonal characteristic due to the distinctive tone used to attract attention, but only if both dominate the noise impact and are not simply audible in the ambient noise. A tonal characteristic would not be applied to a noise that only exhibited tonality at an instant in time. An example is the noise from a wailing siren. At any instant in time, a wailing siren exhibits a tone, but this tone moves across the frequency spectrum over time to generate a modulating noise. |
| Impulsive | Impulsive noise characteristics can be described as the sudden onset of a sound. A noise source that attracts an impulsive characteristic will often be described as something with a thumping, banging or impact noise that is clearly audible above everything else. It is distinguished by a sharp rise in noise level. Some examples include the noise from a heavy mechanical press that consistently produces an impact noise as it stamps out a metal template, the noise from the dropping of material that causes a short burst of loud sound as the material hits the ground, or the noise of heavy hammering and banging from a workshop area. An impulsive characteristic would not be applied to a noise that simply varies in level. The modulating characteristic is established to deal with such a noise. A short burst or impact, or series of impacts, need to be present to apply the impulsive characteristic. |
| Modulating | A noise source that attracts a modulating characteristic will often be described as something with a cyclical, varying, fluctuating, pulsating, or changing noise characteristic that is clearly audible above everything else. Modulation can occur to the measured noise level of a source. The most common case for noise with such characteristics is moving equipment that are out of balance (such as a fan) or in the case of a forklift moving about on a loading zone. It can also occur due to a change to the frequency content, as in the case of a wailing siren. Modulation noise characteristics would also apply to regular truck movements coming in and out of a subject site. The modulation, or rise and fall of the sound, can also occur over a short period, such as the bark from an engine brake, which rises and falls in level many times over a one-second period, or the noise experienced when the rear window of a car is left down, and the speed of the vehicle is sufficient such that the wind noise produces a pulsing sound. |
| Intermittent(2) | Subjectively, a noise source has intermittent noise characteristics if it is described as a noise that comes and goes, or starts and stops. This could apply to a generator that operates sporadically (starts and stops multiple times) during an assessment period. The intermittent noise characteristics may also be applicable for situations when the noise characteristics themselves are intermittent in nature. |

      - The descriptions for the subjective tests are directly from the Guidelines.

Only applicable during the nighttime period (10 pm to 7 am).

These characteristics are considered in the assessment of the predicted noise levels at the receivers. Refer to Section 41 for further information.

## Emergency equipment

The development is to include equipment for fire protection and two standby generators. These items of equipment are only to be tested periodically during the day and would only operate in emergency situations when there is a fire or mains power is down. As such, the strict application of the Noise Policy criteria may be considered to be too onerous

The Guidelines for the use of the Environment Protection (Commercial & Industrial Noise) Policy 2023 (the Guidelines) provides guidance on reasonable adjustments for noise sources that are intermittent and infrequent. Table 8 outlines the allowable adjustment of the criteria for intermittent and infrequent noise sources. Note that the compliance criterion is 5 dB higher than the criteria outlined in Table 7.

Table 8 Allowable adjustment of the criteria

| Allowable duration of noise (1 event in a 24-hour period) | Allowable exceedance of the compliance criterion (INL), dB(A) |  |
| --- | --- | --- |
|  | Day (7 am to 10 pm) | Night (10 pm to 7 am) |
| Under 2 hours | 3 | 0 |
| Under 1 hour | 5 | 0 |
| Under 15 minutes | 10 | 3 |

For the fire protection equipment, we understand that the following operation is typical:

Tested monthly for up to 15 minutes

Tested during daytime only

During emergency (fire).

For the standby generator, we understand that the following operation is typical:

Monthly for up to 30 minutes run time

During emergency (mains power outage).

The criteria as outlined in Table 9, has been adopted for emergency equipment based on the above and the associated rationale.

Table 9 Emergency equipment criteria & rationale

| Equipment | Design criterion, Leq dB(A) | Rationale |
| --- | --- | --- |
| Fire pump, Smoke Spill Fans, Stair Pressurisation Fans, Stair Pressurisation Relief fans | Day INL+10 | Testing of the fire pump will only occur during the daytime period, once a month, for a 15-minute period. The daytime exceedance is consistent with Table 8. The nighttime criterion will be exceeded when the fire pump is in operation; however, this will be in the event of a fire when noise emissions and protection of amenity is not typically considered to be the priority. |
| Standby generator, Load bank | Day INL+0 Night INL+7 | Testing of the standby generator will only occur during the daytime period, once a month, for a 30-minute period. The proposed day time exceedance is 5 dB less than what is proposed in Table 8. However, it is 7 dB more than what is proposed for the nighttime criterion. We believe that the adoption of this criterion will be suitable for both day and night. The proposed results in a higher nighttime criterion than proposed by the new guidelines; however, Resonate believe that this will be acceptable based on the infrequent operation of the generator in an emergency. |

Application of the proposed exceedance as detailed in Table 9 results in criteria as shown in Table 10.

Table 10 Summary of emergency equipment criteria

| Receiver | Criteria, Leq dBA) |  |
| --- | --- | --- |
|  | Fire pump | Standby generator |
| Premises to north and west of subject site | 72 | 62 |
| Premises to southwest of subject site along Alford Road | 67 | 57 |
| Premises to east of subject site along Glynburn Road | 70 | 60 |

## Music noise

### EPA criteria

Music noise emissions are typically assessed under the Environment Protection Authority (EPA) Guideline Assessing music noise from indoor venues (2021). This guideline has been developed by the South Australian EPA to assist the assessment of music noise emissions for development applications.

For new music venues, if the development is for a venue at which music is the prime source of entertainment, such as a night club, or is such that music noise is likely to be audible outside the venue, the venue should be designed to achieve the following music noise (LA10,15min) criteria externally at the nearest existing noise sensitive location: "less than 8 dB above the background noise level (L90,15min) in any octave band of the sound spectrum".

The basis of criteria is consistent with DTS/DPF 4.6 of the Planning & Design Code, Interface between Land Uses.

The guideline notes that '…this is a design criterion is for use by an acoustic engineer, and is not intended to be used as a condition of development approval as it is difficult to measure and enforce.'

### Background noise and resultant criteria

The background noise level should be the lowest background noise level measured over a 15-minute period during the time of day when the proposed development will operate.

Background noise monitoring was undertaken at [location] from Friday, 14 to Thursday, 20 June 2013.

The lowest background noise level for Wednesday to Sunday night trading up to 2 am and Monday and Tuesday at 12 midnight is 40 dB(A). The lowest background spectral levels are presented in Table 11.

Table 11 Lowest measured background noise level—2 am Thursday to Monday mornings

| Noise level, dB(Lin) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octave band centre frequency, Hz |  |  |  |  |  |  |
| 63 | 125 | 250 | 500 | 1000 | 2000 | 4000 |
| 46 | 44 | 41 | 37 | 35 | 29 | 20 |

For Monday and Tuesday night trading from 12 midnight to 2 am, the lowest background noise level is 37 dB(A). The lowest background spectral levels are presented in Table 12.

Table 12 Lowest measured background noise level—2 am Tuesday and Wednesday mornings

| Noise level, dB(Lin) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octave band centre frequency, Hz |  |  |  |  |  |  |
| 63 | 125 | 250 | 500 | 1000 | 2000 | 4000 |
| 41 | 40 | 41 | 35 | 31 | 26 | 17 |

Based on the measured background noise levels, the L10 music noise criteria as outlined in Table 13are applicable.

Table 13 Applicable music noise criteria

| Music noise criteria, L10 | Noise level, dB(Lin) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octave band centre frequency, Hz |  |  |  |  |  |  |
|  | 63 | 125 | 250 | 500 | 1000 | 2000 | 4000 |
| Monday & Tuesday night trading from 12 midnight to 2 am | 49 | 48 | 49 | 43 | 39 | 34 | 25 |
| All other proposed operating times | 54 | 52 | 49 | 45 | 43 | 37 | 28 |

## Waste collection

The Local Nuisance and Litter Control Act 2016 (LNLC Act) provides a means to assess noise from waste collection at a commercial premises.

The requirements for under the LNLC Act for this source are outlined in Table 8.

Table 14 Requirements of the LNLC Act

| Description | Example noise sources | Nuisance trigger |
| --- | --- | --- |
| Waste collection | Waste collection | If it is considered that that the noise has travelled from the place at which it was generated to neighbouring premises: - before 9am or after 7pm on any Sunday or public holiday; or - after 7pm or before 7am on any other day. |

With respect to noise, the LNLC Act was written to utilise subjective compliance assessments and therefore does not include any specific noise goals.

We note that the LNCL Act is used to assess whether an existing noise source constitutes a local nuisance. It is not intended to be used during the planning stage. However, the LNLC Act requirements regarding noise associated with waste collection are to provide a restriction on the hours on when the waste collection activities to be occurred once the development is operational and should be considered at the planning stage.

# Domestic—Noise criteria

The Local Nuisance and Litter Control Act 2016 (LNLC Act) provides a means to assess noise from residential premises, and considers the following sources that may be present due to a residence:

fixed machine noise on a domestic premises

noise other than fixed machine noise on a domestic premises

waste collection.

The requirements for under the LNLC Act for these sources are outlined in Table 8.

Table 15 Requirements of the LNLC Act

| Description | Example noise sources | Nuisance trigger |
| --- | --- | --- |
| Fixed machine noise on a domestic premises | - External condenser units - Heat pumps - Exhaust fans - Pool pumps | If it is considered that the noise has travelled from the domestic premises to a habitable room, or an outdoor courtyard or entertainment area, on neighbouring premises. |
| Noise other than fixed machine noise on a domestic premises | - Non-fixed machinery - Tools - Equipment | If it is considered that that the noise has travelled from the domestic premises to neighbouring premises between the hours of: - 8 pm to 9 am on Sunday; or - 8 pm to 8 am on any other day. |
| Waste collection | Waste collection | If it is considered that that the noise has travelled from the domestic premises to neighbouring premises: - before 9am or after 7pm on any Sunday or public holiday; or - after 7pm or before 7am on any other day. |

With respect to noise, the LNLC Act was written to utilise subjective compliance assessments and therefore does not include any specific noise goals. The general provisions of the Environment Protection (Commercial and Industrial Noise) Policy 2023 (Noise Policy) may be used to supplement compliance assessments undertaken by local council under the LNLC Act. This primarily relates to 'fixed machine noise'.

We note that the LNLC Act is used to assess whether an existing noise source constitutes a local nuisance. It is not intended to be used during the planning stage. However, as the objective criteria within the Noise Policy can be adopted to supplement compliance assessments, we suggest that consideration of building services against the Noise Policy criteria at planning stage will demonstrate the ability for the noise sources to comply once operational.

The environmental noise criteria are based on the land use categories determined based on the principal land uses outlined in the Planning & Design Code for the noise source and noise-affected premises. The land uses and indicative noise factors are as shown in Table 6.

Table 6 Excerpt from Noise Policy—Table 2(subclause(1)(b))

| Land use category | Indicative noise factor dB(A) |  |
| --- | --- | --- |
|  | Day (7 am to 10 pm) | Night (10 pm to 7 am) |
| Rural living | 47 | 40 |
| Residential | 52 | 45 |
| Rural industry | 57 | 50 |
| Light industry | 57 | 50 |
| Commercial | 62 | 55 |
| General industry | 65 | 55 |
| Special industry | 70 | 60 |

Based on the land use categories as outlined in the EPA’s Indicative Noise Factor Guidelines, the relevant criteria for the noise-affected premises in each zone are as outlined in Table 7. In accordance with Part 4 of the Noise Policy, the relevant criteria are the average of the relevant indicative noise factors.

Table 7 Summary of zones, land uses, and Noise Policy criteria

| Site | Zone/subzone | Land use categories | Criteria, LAeq |  |
| --- | --- | --- | --- | --- |
|  |  |  | Day (7 am to 10 pm) | Night (10 pm to 7 am) |
| Subject site | Residential | Residential | N/A | N/A |
| Premises to the south along Mascotte Street | Employment | Light Industry & Commercial | 55 | 48 |
| Premises to the west along Victoria Road | Neighbourhood zone | Residential & Commercial | 53 | 46 |

## Emergency equipment

The development is to include equipment for fire protection and two standby generators. These items of equipment are only to be tested periodically during the day and would only operate in emergency situations when there is a fire or mains power is down. As such, the strict application of the Noise Policy criteria may be considered to be too onerous

The Guidelines for the use of the Environment Protection (Commercial & Industrial Noise) Policy 2023 (the Guidelines) provides guidance on reasonable adjustments for noise sources that are intermittent and infrequent. Table 8 outlines the allowable adjustment of the criteria for intermittent and infrequent noise sources. Note that the compliance criterion is 5 dB higher than the criteria outlined in Table 7.

Table 8 Allowable adjustment of the criteria

| Allowable duration of noise (1 event in a 24-hour period) | Allowable exceedance of the compliance criterion (INL), dB(A) |  |
| --- | --- | --- |
|  | Day (7 am to 10 pm) | Night (10 pm to 7 am) |
| Under 2 hours | 3 | 0 |
| Under 1 hour | 5 | 0 |
| Under 15 minutes | 10 | 3 |

For the fire protection equipment, we understand that the following operation is typical:

Tested monthly for up to 15 minutes

Tested during daytime only

During emergency (fire).

For the standby generator, we understand that the following operation is typical:

Monthly for up to 30 minutes run time

During emergency (mains power outage).

The criteria as outlined in Table 9, has been adopted for emergency equipment based on the above and the associated rationale.

Table 9 Emergency equipment criteria & rationale

| Equipment | Design criterion, Leq dB(A) | Rationale |
| --- | --- | --- |
| Fire pump, Smoke Spill Fans, Stair Pressurisation Fans, Stair Pressurisation Relief fans | Day INL+10 | Testing of the fire protection equipment will only occur during the daytime period, once a month, for a 15-minute period. The daytime exceedance is consistent with Table 8. The nighttime criterion will be exceeded when the fire pump is in operation; however, this will be in the event of a fire when noise emissions and protection of amenity is not typically considered to be the priority. |
| Standby generator, Load bank | Day INL+0 Night INL+7 | Testing of the standby generator will only occur during the daytime period, once a month, for a 30-minute period. The proposed day time exceedance is 5 dB less than what is proposed in Table 8. However, it is 7 dB more than what is proposed for the nighttime criterion. We believe that the adoption of this criterion will be suitable for both day and night. The proposed results in a higher nighttime criterion than proposed by the new guidelines; however, Resonate believe that this will be acceptable based on the infrequent operation of the generator in an emergency. |

Application of the proposed exceedance as detailed in Table 9 results in criteria as shown in Table 10.

Table 10 Summary of emergency equipment criteria

| Receiver | Criteria, Leq dBA) |  |
| --- | --- | --- |
|  | Fire pump | Standby generator |
| Premises to north and west of subject site | 72 | 62 |
| Premises to southwest of subject site along Alford Road | 67 | 57 |
| Premises to east of subject site along Glynburn Road | 70 | 60 |

# Noise monitoring

## Details

Background noise monitoring [location of noise monitoring] was undertaken from [date and time of monitoring].

## Instrumentation

The noise measurements were taken with a calibrated Rion NL-52 sound level meter, which is a Class 1 instrument suitable for field and laboratory use. The sound level meter was calibrated both before and after the measurements using a Class 1 Brüel & Kjær 4231 sound level calibrator, and the calibration was found to have not drifted. Both the sound level meter and calibrator carry current calibration certificates from a NATA accredited laboratory. Copies of the calibration certificates are available on request.

## Procedure

Noise measurements were undertaken in accordance with the following:

The microphone of the sound level meter was at a height of approximately 1.2 metres above the ground and at least 3.5 metres away from any wall or facade.

The axis of maximum sensitivity of the microphone of the sound level meter was directed towards the noise source.

A wind shield was used during all measurements, and the measurements were undertaken during a calm, still night (for which the wind velocity did not exceed 5 m/s).

Care was taken to avoid any effect on the measurement of extraneous noise, acoustic vibration or electrical interference.

Noise measurements were undertaken for a period of 5 – 10 minutes. These time periods were chosen as it was noted on location that the noise from the Simsmetal site did not vary significantly over the period and the measured Leq levels were relatively constant over the measurement period.

## Results

The results of the noise monitoring are shown in Appendix …. graph of measured levels or table summarising relevant noise descriptors.

Discussion of relevant aspects of measured background noise levels—relative to the proposed opening hours of the proposed development.

Comment on the logged levels wtr to the criteria if relevant.

Note the following requirements from the Guidelines:

# Assessment

## Noise modelling

### Modelling parameters

ISO 9613-2:1996

Modelling was undertaken in SoundPLAN Environmental Software v8.2 program, using the ISO 9613-2:1996 algorithm.

This method predicts sound pressure levels under meteorological conditions favourable to propagation from noise sources. These conditions are for downwind propagation, or, equivalently, propagation under a well-developed moderate groundbased temperature inversion, such as that which commonly occurs at night.

Specific terms are provided in the algorithm for the following physical effects:

geometrical divergence

atmospheric absorption

ground effect

reflection from surfaces

screening by obstacles.

CONCAWE

Noise emissions from site have been modelled in SoundPLAN Environmental Software v8.2 program, using the Conservation of Clean Air and Water in Europe (CONCAWE) algorithms. The model takes into consideration:

attenuation of noise source due to distance

barrier effects from buildings, topography and the like

air absorption

ground effects

weather conditions (wind speed, wind direction, time of day, and cloud cover)

CONCAWE has six difference meteorological categories—CONCAWE meteorological category 1 represents weather conditions that are least conducive to noise propagation (best case situation with the lowest predicted noise levels), CONCAWE meteorological category 4 represents neutral weather conditions, and CONCAWE meteorological category 6 represents weather conditions that are the most conducive to noise propagation (the worst case situation with the highest predicted noise levels). In accordance with the Guidelines for the use of the Environmental Protection (Commercial & Industrial Noise) Policy 2023, CONCAWE meteorological category 5 has been used for day time noise emissions and CONCAWE meteorological category 6 has been used for night time noise emissions.

General prediction

Noise emissions from site have been modelled in SoundPLAN Environmental Software v8.2 program, using the general prediction method. The model takes into consideration:

attenuation of noise source due to distance

barrier effects from buildings, topography and the like

air absorption

ground effects

neutral meteorological conditions (zero wind and temperature gradients).

Geometric spreading

Noise emissions from site have been modelled assuming attenuation from geometrical spreading only and neutral meteorological conditions (zero wind and temperature gradients). Other factors such as ground effects, air absorption and shielding due to natural topography have not been considered.

### Modelling scenarios

Modelling scenarios aligned with the operations as outlined in Section 2.2 are presented in Table 18.

Table 18 Modelling scenarios

| Scenarios | Activity | Criteria |
| --- | --- | --- |
| Day—7 am to 10pm |  |  |
| General activity (day time) | - Car park activity - External mechanical plant | Noise Policy day |
| Childcare | - Outdoor childcare play | WHO moderate annoyance |
| General activity (day time) & emergency equipment testing | - Car park activity - External mechanical plant - Standby generator | Noise Policy day + 5 |
| - Night—10pm to 7 am |  |  |
| General activity (nighttime) | - External mechanical plant | Noise Policy night |

## Sound levels

Sound levels adopted in the noise modelling of the proposed development are outlined in Table 19.

Table 19 Sound levels

| Location | Details | Quantity | Sound level | Notes |
| --- | --- | --- | --- | --- |
| Outside play area 1 | Children, 0 – 2 years | 35 | LW 78 dB(A) per 10 children | 1 |
| Outside play area 2 | Children, 2 – 3 years | 20 | LW 85 dB(A) per 10 children | 1 |
| Outside play area 3 | Children, 3 – 5 years | 15 | LW 87 dB(A) per 10 children | 1 |
| xx | Small (single fan) condenser | 1 | LW 65 dB(A) | 1 |
| xx | Medium (double fan) condenser | 1 | LW 70 dB(A) | 1 |
| xx | Large (double fan) condenser | 1 | LW 80 dB(A) | 1 |
| xx | Small exhaust fan (toilet, garbage room) | 1 | LW 60 dB(A) | 1 |
| xx | Small kitchen exhaust fan | 1 | LW 70 dB(A) | 1 |
| xx | Carpark exhaust fan | 1 | LW 85 dB(A) | 1 |
| Carpark | Car / passenger vehicles | 5 | LW 81 dB(A) | 1 |
| Carpark | Delivery van | 1 | LW 86 dB(A) | 1 |
| Clubroom function & deck areas | Patrons | 200 | LW 74 dB(A) per patron | 2 |
| Clubroom function room | Background music | — | Lp 73 dB(A) L10 | 2,3,4 |
|  | Acoustic amplified music | — | Lp 83 dB(A) L10 | 2,3 |
|  | Ambient DJ / Friday night front bar | — | Lp 88 dB(A) L10 | 2,3 |
|  | Function room (wedding etc) | — | Lp 93 dB(A) L10 | 2,3 |
|  | Friday / Saturday night pub dance floor | — | Lp 98 dB(A) L10 | 2,3 |
|  | Night club | — | Lp 113 dB(A) L10 | 2,3 |
| Carpark | Passenger vehicle movements | 40 | LW / m2 70 dB(A) | 5 |
| Loading dock | Medium rigid vehicle | 1 | LW 99 dB(A) engine LW 91 dB(A) exhaust | 6 |
|  | Refrigerated truck unit | 1 | LW 92 dB(A) | 6 |
|  | Electric forklift | 1 | LW 84 dB(A) | 6 |
| Workshop / store roof | Evaporative cooler | 6 | Radiated LW 73 dB(A) | 7 |
|  | General exhaust fan | 3 | Outlet LW 99 dB | 7 |
| Office roof | Small condensers | 6 | Lp 50 dB(A) @ 1m | 7 |
|  | Larger condenser | 1 | Radiated LW 93 dB(A) | 7 |
|  | Larger condenser | 1 | Radiated LW 83 dB(A) | 7 |
|  | Larger condenser | 1 | Radiated LW 80 dB(A) | 7 |
| Mezzanine roof open plant area | Heat pump | 3 | Lp 73 dB(A) @ 1m | 7 |
|  | Domestic hot heat pump | 2 | Lp 61 dB(A) @ 3m | 7 |
| Mezzanine roof enclosed plant area | Air Handling Units | 4 | Radiated LW 80 dB(A) | 7 |
|  | Air compressor | 1 | Lp 74 dB(A) @ 1m | 7 |
|  | Dryer | 1 | Lp 60 dB(A) @ 3m | 7 |

      - Association of Australasian Acoustical Consultants Guideline for Child Care Centre Acoustic Assessment V3.

Association of Australasian Acoustical Consultants SA Draft Guide for Music Venues and Outdoor Areas.

For music spectrum adopted refer to Table 21.

For background music only, no assessment is required as noise will be controlled by patron noise.

Parking Area Noise Recommendations for the Calculation of Sound Emissions of Parking Areas, Motorcar Centers and Bus Stations as well as of Multi-Storey Car Parks and Underground Car Parks 6 Revised Edition, Bayerisches Landesamt für Umwelt, 2007, document.

Resonate database of measured and researched data.

Provided by the Mechanical Engineer.

For the function room, it is assumed that music noise levels similar to that typically played during a wedding function or similar will be desired. As such, an internal music sound pressure level of 93 dB(A) L10 has been adopted with spectral data as outlined in Table 21.

Table 21 Allowable music noise level

| Noise level, L10 dB(Lin) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octave band centre frequency, Hz |  |  |  |  |  |  |
| 63 | 125 | 250 | 500 | 1000 | 2000 | 4000 |
| 95 | 90 | 90 | 90 | 90 | 85 | 85 |

## Recommended treatments

### Building services noise

At this stage of the development process, only indicative plant selections have been provided. The selections and location of plant are subject to change as the design develops. Sound power levels, noise attenuation, and enclosures or barriers are to be designed during detailed design stage to ensure compliance with required noise emission levels.

Acoustic treatments that may have to be considered are outlined in Table 22.

Table 22 Indicative acoustic treatments for building services

| Plant and/or location | Potential noise mitigation options |
| --- | --- |
| Plant selection & operation | - Low noise options, in particular for roof top chillers with the Option 15LS and 15LS+ to be considered - Nighttime setback mode where possible |
| Enclosed plant rooms | - Absorptive treatment to internal plant room walls and ceiling - Solid roof and walls - Acoustic louvres (300 – 600 mm deep), such as Fantech Sound Bar Louvres SBL1 or SBL2 |
| Roof top plant areas | - Acoustic louvres (300 – 600 mm deep), such as Fantech Sound Bar Louvres SBL1 or SBL2 |
| - Exhaust fans Smoke Spill fans | - Acoustically lined ductwork - Discharge or intake attenuators (1 or 2 diameters long), such as Fantech C1 or C2 series. |
| Diesel generators | - Absorptive treatment to internal generator room walls - Exhaust silencer - Attenuated intake to the generator room - OR - Acoustically treated enclosed generator set |
| Fire pump room | - Attenuated intake and exhaust - Acoustically treated walls and doors |

Static insertion loss values for acoustic louvres are outlined in Table 23.

Table 23 Static insertion loss for acoustic louvres

| Acoustic louvres | Static Insertion Loss, centre band frequency (Hz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 63 Hz | 125 Hz | 250 Hz | 500 Hz | 1000 Hz | 2000 Hz | 4000 Hz |
| Fantech SBL1 | 4 | 7 | 9 | 13 | 14 | 12 | 12 |
| Fantech SBL2 | 5 | 10 | 14 | 22 | 27 | 25 | 21 |

Louvres around plant areas should be orientated to reduce direct line of sight from the receiver to the plant area. Generally this will mean that the louvre blades should be angled up from the plant room side to the receiver side. Refer to Figure 5 for an example.

Figure 5 Louvre orientation

Suitable acoustic absorption products for different plant areas is provided in Table 24.

Table 24 Recommended absorptive treatments

| NRC | Panel |
| --- | --- |
| Internal plant rooms |  |
| 0.95 | Martini Absorb HD 50 |
|  | Bradford Supertel 75 Acoustituff |
| 1.0 | Megasorber FM50 – 50 mm |
|  | Martini Absorb HD 75 |
|  | Bradford Supertel 75 HD Perf / Unfaced |
| External plant areas |  |
| 0.9 | Pyrotek 50 mm Reapor |
| 1.0 | Megasorber 50 mm FM-DS (thermoset acoustic foam with water-repellent Soundmesh G8) |
|  | Stratocell Whisper |

### Loading docks

To control noise from the loading dock areas, the following treatments and operational limits may have to be considered:

1 delivery with refrigerated HRV per 15-minute period in each dock

Trucks are not to idle while unloading

Electric forklift or hand pallet jack to be used for unloading

All trucks waiting for entry to the loading dock to be turned off while stationary.

### Function room

Recommended acoustic treatments that may have to be considered to control music noise emissions from the site are outlined in Table 25.

Table 25 Indicative treatments for the function room

| Item | Indicative treatment |
| --- | --- |
| External facade | Glass to be acoustically equivalent to RW 40 proprietary glass such as 12.5 mm VLam Hush. |
| External doors | RW 30 doors: Glazed doors with perimeter door seals to create an airtight seal around the perimeter of all doors and should be equivalent to: - Framed 12.5 mm VLam Hush (or other RW 40 glazing) - Raven RP71Si meeting stile seals - Raven RP10Si head and jamb seals - RP70Si bottom seals Airlock entry for entry / exit during playing of amplified music during late night functions formed with 2x RW 30 doors. |
| Airlock | - Airlock entry for entry / exit during playing of amplified music during late night functions - Airlock to be formed with 2 x RW 30 doors - Ensure one set of doors is closed at all times. |
| External walls | - Metal sheet - 92 mm Steel stud - 75 mm, 14 kg/m3 insulation - 2x16 mm fire rated plasterboard (12.5 kg/m2) |
| Music noise limit | - Limit the music noise within the venue to L10 93 dB(A) with a spectrum as shown in Table 21. - The music noise can be limited using a system similar to the Sentry Sound limiters (https://www.waveformacoustics.com.au/noise-and-sound-limiters). |
| Roof/ceiling | - Metal deck roof - 500 mm ceiling cavity - 75 mm, 14 kg/m3 insulation - 2x13 mm sound rated plasterboard - Any feature ceiling to be under the acoustic ceiling |
| Internal absorption | To control reverberant noise levels within the pavilion, acoustic absorption (equivalent to an NRC 0.7 ceiling) is required. Note that this will be required to provide a suitable internal space for the various proposed uses. |
| Operational hour limit | Limit hours of operation to midnight every night. |

## Predicted noise levels

### General activity

A summary of the predicted noise levels at noise-affected premises is presented in Table 26 and Table 27 for day and night time emissions respectively.

Table 26 Predicted noise levels—day

| Prediction location | Predicted noise level, Leq dB(A) | Noise Policy day time criteria, dB(A) |
| --- | --- | --- |
| Premises to the north along Maple Avenue | ≤ 50 + 5 = 55(1) | 57 |
| Premises to the east | ≤ 45 | 55 |
| Premises to the south along Leader Street | ≤ 49 | 52 |
| Premises to the west across Anzac Highway | ≤ 45 | 55 |

A 5 dB(A) penalty may be applicable to account for modulating or impulsive noise associated with manufacturing activities. The presented level includes a 5 dB(A) penalty.

Table 27 Predicted noise levels—night

| Prediction location | Predicted noise level, Leq dB(A) | Noise Policy nighttime criteria, dB(A) |
| --- | --- | --- |
| Premises to the east | ≤ 40 + 5 = 45(1) | 48 |
| Premises to the south along Leader Street | ≤ 40 | 45 |
| Premises to the west across Anzac Highway | ≤ 40 | 48 |

      - A 5 dB(A) penalty may be applicable to account for modulating or impulsive noise associated with manufacturing activities. The presented level includes a 5 dB(A) penalty.

### Music noise

The highest predicted music noise levels at the residential premises are presented in Table 28.

Table 28 Predicted music noise levels compared to criteria

|  | Noise level, dB(Lin) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octave band centre frequency, Hz |  |  |  |  |  |  |
|  | 63 | 125 | 250 | 500 | 1000 | 2000 | 4000 |
| Predicted music noise level, L10 | 57 | 50 | 45 | 39 | 33 | 26 | 15 |
| Music noise criteria, L10 | 57 | 53 | 48 | 46 | 44 | 38 | 29 |

### Summary

The predicted indicative noise levels comply with the Noise Policy and music noise criteria based on:

proposed scenarios as outlined in Section 2.2, and

treatments as outlined in Section 6.2.5.

## Waste collection

Noise from waste collection is not addressed under the Noise Policy. The Local Nuisance and Litter Control Act 2016 (LNLC Act) provides guidance in this case and notes that if waste collection is restricted to 9 am and 7 pm on a Sunday or public holiday and 7 am and 7 pm on any other day, noise from this activity will not be considered to create a noise nuisance. If waste collection is restricted to the noted times, there will be no further restrictions.

# Conclusion

An environmental noise impact assessment has been undertaken for the proposed [development name] at [location].

This assessment has demonstrated that, with the indicative noise mitigation treatments detailed in this report the noise emissions from the operation of the proposed development will be able to comply with the relevant documents:

Planning & Design Code

Environment Protection (Commercial & Industrial Noise) Policy 2023 (Noise Policy)

Local Nuisance and Litter Control Act 2016

World Health Organization (WHO) Guidelines for Community Noise

Environment Protection Authority (EPA) Guideline Assessing music noise from indoor venues (2021).

All recommended treatments will be further reviewed and refined during the detailed design of the development, and as such, all recommendations provided within the report are to be considered as indicative only.

Guidelines for Community Noise, World Health Organization, Geneva. Published April 1999.